4 Port HDMI Splitter

AC-1710

User manual

- Introduction

1×4 HDMI splitter distributes 1 HDMI source to 4 HDMI displays simultaneously. It offers HD video solution for HDTV/STB/DVD/Projector/DVR in homes, retail, conference room presentation, education and training, etc.

  - Features

- 1 HDMI input signal split to 4 HDMI  devices.

- HDCP 1.4 protocol compliant.

- 3D HDMI 1.4a video format support.

- 3D video support including Frame packing for all 3D formats up to a 297MHz TMDS clock.

- Support display resolutions up to 4K x 2K@30Hz,1080P@120Hz,and 1080P 3D@60Hz

- Support deep color 30bit, 36bit, 48bit per pixel.

- Digital audio format, as DTS-HD/Dolby-true-HD/LPCM 7.1/DTS/Dolby-AC3/DSD.

  - Package

- HDMI Splitter ----------------------------------------------------------------------------- 1PCS

- 5V1A DC Power Supply adaptor ------------------------------------------------------- 1PCS

- Operation Manual ------------------------------------------------------------------------- 1PCS

  - Specifications

- Frequency Bandwidth                2.97Gbps

- HDMI Splitter Input Ports             1 x HDMI Female input port

- HDMI Splitter Output Ports            4 x HDMI Female output ports

- Power Supply                       DC 5V 1A

- ESD Protection Human Body Model:    ± 8kV (air-gap discharge)

± 4kV (contact discharge)

- Dimensions (mm)                   70(W) X 145 (D) X 25 (H)

  - Operation controls and Functions

Front Panel

- ON/OFF: Power on/off switch.

- POWER LED: This red LED illuminates when the device is connected with power supply.

- INPUT LED: This red LED illuminates when a HDMI signal from a source is plugged in.

- OUTPUT LED: These red LED illuminates with the corresponding HDMI output port.

Rear Panel

- HDMI OUTPUT: These sockets connect to a HDTV or monitor with a HDMI cable.

- HDMI INPUT: This socket connects to a HDMI source output, i.e. from a DVD player, Game console, etc.

- DC 5V: Plug the 5V DC power supply into the unit and connect the adaptor to AC wall outlet.

  - Application Example

IMPORTANT:

- Never attempt to open, repair or modify any part of this unit or its accessories.

- Never get any part of the device or its accessories wet.

- Failure to adhere to these points will result in a void of warranty.